digital lock system

Final Project

Digital Logic Laboratory

Table of Contents

Table of Tables	2

Table of Figures	2

Abstract	3

Introduction	3

Design	3

Equipment and Programs	3

Block Diagram and Module Break Down	3

Top Module	3

Buttons	4

Block Diagram	4

Code	5

Counter Module	6

Code	6

Display Module	6

Code	7

Finite State Machine (FSM) Module	9

States	9

Reset	9

State Diagram	10

Table Diagram	10

Code	11

Constraints	17

Code	17

Test Bench	19

Scenarios	19

Results	20

Conclusion	22

# Table of Tables

Table 1 Table Diagram of FSM	10

# Table of Figures

Figure 1 button representation on Nexys 4 Board	4

Figure 2 Top module schematic	4

Figure 3 State Diagram of FSM	10

Figure 4 Right from the first try	19

Figure 5 Right from second try	19

Figure 6 Wrong twice	20

Figure 7 Output 1	20

Figure 8 Output 2	21

Figure 9 Output 3	22

# Abstract

Design a three digit combinational digital lock and implement it using the Nexys4 board. The lock system has five input buttons each representing an integer number, two led lights representing the two states of the system “locked” and “unlocked” and a seven segment display that displays the entered input and number of tries. To unlock the system, the user should enter the combinational lock key “241”. He will have two tries to enter the right key or else the system will stay locked.

# Introduction

The lock system is comprised of three main modules; Counter module, display module, and finite state machine (FSM) module. The system can be in two states “locked” and “unlocked”. The top module is responsible for the connections between the three main modules and the inputs and outputs of the system. The test-bench module tests different inputs/outputs of the finite state machine (FSM) module.

# Design

## Equipment and Programs

- Nexys 4 board

- Vivado Design Suite (VDS) software

- A computer

## Block Diagram and Module Break Down

## Top Module

This module pieces all of the other modules together to get the full lock system. It receives information directly from the push buttons (BTNU / BTNR / BTNC BTNL / BTND) and passes them to the FSM module. It connects the clock received from the system to the Counter Module. It manages the inputs and outputs between the components. It passes the output from the display module to the seven segment display.

### Buttons

Figure 1 button representation on Nexys 4 Board

Each button represents a digit

- BTNU = 1

- BTNL = 2

- BTNC = 3

- BTNR = 4

- BTND = 5

### Block Diagram

Figure 2 Top module schematic

### Code

module digital_lock (

input  wire       clk   ,

input  wire       reset ,

input  wire       d1    ,

input  wire       d2    ,

input  wire       d3    ,

input  wire       d4    ,

input  wire       d5    ,

output wire       unlock,

output wire       lock  ,

output wire [3:0] an    ,

output wire [7:0] sseg

);

wire [2:0] disp_1_wire ;

wire [2:0] disp_2_wire ;

wire [2:0] disp_3_wire ;

wire [2:0] disp_N_wire ;

wire       new_clk_wire;

Cntr_Unit Cntr (.clk(clk), .reset(reset), .new_clk(new_clk_wire));

FSM_lock_Unit FSM_lock (

.clk   (new_clk_wire),

.lock  (lock        ),

.unlock(unlock      ),

.disp_1(disp_1_wire ),

.disp_2(disp_2_wire ),

.disp_3(disp_3_wire ),

.disp_N(disp_N_wire ),

.reset (reset       ),

.d1    (d1          ),

.d2    (d2          ),

.d3    (d3          ),

.d4    (d4          ),

.d5    (d5          )

);

Disp_Unit Disp (

.clk   (clk        ),

.reset (reset      ),

.disp_1(disp_1_wire),

.disp_2(disp_2_wire),

.disp_3(disp_3_wire),

.disp_N(disp_N_wire),

.sseg  (sseg       ),

.an    (an         )

);

endmodule

## Counter Module

This module slows down the FPGA internal clock from 100MHz to 10Hz. It takes the system’s clock as input and according to a counter register it either outputs “HIGH” when it reaches a value of 10,000,000 or “LOW” otherwise.

### Code

module Cntr_Unit (

input      clk    ,

input      reset  ,

output reg new_clk

);

reg [31:0] r_reg;

always @ (posedge clk, posedge reset)begin

if (reset) begin

r_reg   <= 0;

new_clk <= 0;

end else begin

if (r_reg == 50000000) begin //10000000

new_clk <= ~new_clk;

r_reg   <= 0;

end else begin

new_clk <= new_clk;

r_reg   <= r_reg + 1;

end

end

end

endmodule

## Display Module

The display module will use the Seven Segment Display (SSD) to display the three values of the pressed buttons and the number of the users tries. The SSD is low level sensitive, meaning they light up when they are “LOW”. The module accepts the binary data for seven segment displays and decodes it using time division multiplexing.

### Code

module Disp_Unit (

input      [2:0] disp_1,

input      [2:0] disp_2,

input      [2:0] disp_3,

input      [2:0] disp_N,

input            reset ,

input            clk   ,

output reg [7:0] sseg  ,

output reg [3:0] an

);

localparam   N       = 18;

reg  [N-1:0] q_reg       ;

wire [N-1:0] q_next      ;

reg  [  2:0] disp_in     ;

always @ ( posedge clk, posedge reset)

if (reset)

q_reg <= 0;

else

q_reg <= q_next;

assign q_next = q_reg + 1;

always @*

case (q_reg [N-1:N-2])

2'b00 :

begin

an      = 4'b1110;

disp_in = disp_1;

end

2'b01 :

begin

an      = 4'b1101;

disp_in = disp_2;

end

2'b10 :

begin

an      = 4'b1011;

disp_in = disp_3;

end

default :

begin

an      = 4'b0111;

disp_in = disp_N;

end

endcase

always @*

begin

case (disp_in)

3'b000  : sseg[6:0] = 7'b1000000 ;

3'b001  : sseg[6:0] = 7'b1111001 ;

3'b010  : sseg[6:0] = 7'b0100100 ;

3'b011  : sseg[6:0] = 7'b0110000 ;

3'b100  : sseg[6:0] = 7'b0011001 ;

3'b101  : sseg[6:0] = 7'b0010010 ;

default : sseg[6:0] = 7'b1000000 ;

endcase

sseg[7] = 1'b1;

end

endmodule

## Finite State Machine (FSM) Module

### States

The FSM has eight states:

### Reset

Initializes the system and sets the state register to the initial state “A”.

#### State A

- Resets everything but the number of tries to the following initial conditions:

- Unlock led turned off

- Lock led turned on

- First display to zero

- Second display to zero

- Third display to zero

- It transitions into state “B” if the first input was equal to “2” or state “E” otherwise

#### State B

- sets the first display to the first input value

- It transitions into state “C” if the second input was equal to “4” or state “F” otherwise

#### State C

- sets the second display to the second input value

- It transitions into state “D” if the third input was equal to “1” or state “G” otherwise

#### State D

- sets the third display to the third input value

- Sets the unlock led to “HIGH”

- The system stays at this state

#### State E

- sets the first display to the first input value

- It transitions into state “F” once the second input is entered

#### State F

- sets the second display to the second input value

- It transitions into state “G” once the third input is entered

#### State G

- sets the third display to the third input value

- increments the number of tries “N”

- It transitions into state “H” if the number of tries “N” is less than two or state “A” otherwise.

#### State H

- sets lock led to “HIGH”

- The system stays at this state

### State Diagram

Figure 3 State Diagram of FSM

### Table Diagram

| Current state | Next State |  |  |  |  |  |  |  |  |  | Output |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | d_1 |  | d_2 |  |  | d_3 |  |  | N |  | disp_1 | disp_2 | disp_3 | N | unlock | lock |
|  | 2 | !2 | 4 | !4 | X | 1 | !1 | X | <=1 | >1 |  |  |  |  |  |  |
| A | B | E | - | - | - | - | - | - | - | - | 0 | 0 | 0 | N | 0 | 0 |
| B | - | - | C | F | - | - | - | - | - | - | d_1 | 0 | 0 | N | 0 | 0 |
| C | - | - | - | - | - | D | G | - | - | - | d_1 | d_2 | 0 | N | 0 | 0 |
| D | - | - | - | - | - | - | - | - | - | - | d_1 | d_2 | d_3 | N | 1 | 0 |
| E | - | - | - | - | F | - | - | - | - | - | d_1 | 0 | 0 | N | 0 | 0 |
| F | - | - | - | - | - | - | - | G | - | - | d_1 | d_2 | 0 | N | 0 | 0 |
| G | - | - | - | - | - | - | - | - | H | A | d_1 | d_2 | d_3 | N - 1 | 0 | 0 |
| H | - | - | - | - | - | - | - | - | - | - | d_1 | d_2 | d_3 | N | 0 | 1 |

Table 1 Table Diagram of FSM

### Code

module FSM_lock_Unit (

input            clk   ,

input            d1    ,

input            d2    ,

input            d3    ,

input            d4    ,

input            d5    ,

output reg       lock  ,

output reg       unlock,

output reg [2:0] disp_1,

output reg [2:0] disp_2,

output reg [2:0] disp_3,

output reg [2:0] disp_N,

input            reset

);

parameter [2:0] A = 3'b000, B = 3'b001, C = 3'b010, D = 3'b011,E = 3'b100, F = 3'b101, G = 3'b110, H = 3'b111;

reg [2:0] N         ;

reg [2:0] N_r       ;

reg [2:0] state_next;

reg [2:0] state_reg ;

reg [2:0] disp_1_r;

reg [2:0] disp_2_r;

reg [2:0] disp_3_r;

reg [2:0] disp_N_r;

reg lock_r;

reg unlock_r;

reg d1_r;

reg d2_r;

reg d3_r;

reg d4_r;

reg d5_r;

wire d1_flag;

wire d2_flag;

wire d3_flag;

wire d4_flag;

wire d5_flag;

always @(posedge clk or posedge reset) begin

if(reset) begin

d1_r <= 0;

d2_r <= 0;

d3_r <= 0;

d4_r <= 0;

d5_r <= 0;

end else begin

d1_r <= d1;

d2_r <= d2;

d3_r <= d3;

d4_r <= d4;

d5_r <= d5;

end

end

assign d1_flag = ~d1_r & d1;

assign d2_flag = ~d2_r & d2;

assign d3_flag = ~d3_r & d3;

assign d4_flag = ~d4_r & d4;

assign d5_flag = ~d5_r & d5;

always @ ( posedge clk or posedge reset)

if (reset)

state_reg <= A;

else

state_reg <= state_next;

always @ ( posedge clk  or posedge reset)

if (reset) begin

N_r      <= 2;

disp_1_r <= 0;

disp_2_r <= 0;

disp_3_r <= 0;

disp_N_r <= 2;

unlock_r <= 0;

lock_r   <= 0;

end else begin

N_r      <= N;

disp_1_r <= disp_1;

disp_2_r <= disp_2;

disp_3_r <= disp_3;

disp_N_r <= disp_N;

unlock_r <= unlock;

lock_r   <= lock;

end

always @(state_reg, d1_flag, d2_flag, d3_flag, d4_flag, d5_flag) begin

disp_1     = disp_1_r;

disp_2     = disp_2_r;

disp_3     = disp_3_r;

disp_N     = disp_N_r;

unlock     = unlock_r;

lock       = lock_r;

N          = N_r;

state_next = state_reg;

case (state_reg)

A :

begin

disp_1 = 3'b000;

disp_2 = 3'b000;

disp_3 = 3'b000;

if (d2_flag)

begin

disp_1     = 3'b010;

state_next = B;

end

else

begin

if (d1_flag) begin

disp_1     = 3'b001;

state_next = E;

end else if (d3_flag) begin

disp_1     = 3'b011;

state_next = E;

end else if (d4_flag) begin

disp_1     = 3'b100;

state_next = E;

end else if (d5_flag) begin

disp_1     = 3'b101;

state_next = E;

end

end

end

B :

begin

if (d4_flag)

begin

disp_2     = 3'b100;

state_next = C;

end

else

begin

if (d1_flag) begin

disp_2     = 3'b001;

state_next = F;

end else if (d2_flag) begin

disp_2     = 3'b010;

state_next = F;

end else if (d3_flag) begin

disp_2     = 3'b011;

state_next = F;

end else if (d5_flag) begin

disp_2     = 3'b101;

state_next = F;

end

end

end

C :

begin

if (d1_flag)

begin

disp_3     = 3'b001;

state_next = D;

end

else

begin

if (d4_flag) begin

disp_3     = 3'b100;

state_next = G;

end else if (d2_flag) begin

disp_3     = 3'b010;

state_next = G;

end else if (d3_flag) begin

disp_3     = 3'b011;

state_next = G;

end else if (d5_flag) begin

disp_3     = 3'b101;

state_next = G;

end

end

end

D :

begin

unlock = 1;

lock   = 0;

end

E :

begin

if (d1_flag) begin

disp_2     = 3'b001;

state_next = F;

end else if (d4_flag) begin

disp_2     = 3'b100;

state_next = F;

end else if (d2_flag) begin

disp_2     = 3'b010;

state_next = F;

end else if (d3_flag) begin

disp_2     = 3'b011;

state_next = F;

end else if (d5_flag) begin

disp_2     = 3'b101;

state_next = F;

end

end

F :

begin

if (d1_flag) begin

disp_3     = 3'b001;

state_next = G;

end else if (d4_flag) begin

disp_3     = 3'b100;

state_next = G;

end else if (d2_flag) begin

disp_3     = 3'b010;

state_next = G;

end else if (d3_flag) begin

disp_3     = 3'b011;

state_next = G;

end else if (d5_flag) begin

disp_3     = 3'b101;

state_next = G;

end

end

G :

begin

N      = N_r - 1;

disp_N = N_r - 1;

if (N_r > 1)

state_next = A;

else

begin

state_next = H;

unlock = 0;

lock   = 1;

end

end

H :

begin

unlock = 0;

lock   = 1;

end

endcase

end

endmodule

## Constraints

This is used for Hardware interfacing. It specifies the mapping of the FPGAs pins to other parts on the board and gives them names and some context.

### Code

## Clock signal

set_property -dict { PACKAGE_PIN E3    IOSTANDARD LVCMOS33 } [get_ports { clk }]; #IO_L12P_T1_MRCC_35 Sch=clk100mhz

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports {clk}];

##Switches

set_property -dict { PACKAGE_PIN J15   IOSTANDARD LVCMOS33 } [get_ports { reset }]; #IO_L24N_T3_RS0_15 Sch=sw[0]

## LEDs

set_property -dict { PACKAGE_PIN H17   IOSTANDARD LVCMOS33 } [get_ports { unlock }]; #IO_L18P_T2_A24_15 Sch=led[0]

set_property -dict { PACKAGE_PIN K15   IOSTANDARD LVCMOS33 } [get_ports { lock }]; #IO_L24P_T3_RS1_15 Sch=led[1]

##7 segment display

set_property -dict { PACKAGE_PIN T10   IOSTANDARD LVCMOS33 } [get_ports { sseg[0] }]; #IO_L24N_T3_A00_D16_14 Sch=ca

set_property -dict { PACKAGE_PIN R10   IOSTANDARD LVCMOS33 } [get_ports { sseg[1] }]; #IO_25_14 Sch=cb

set_property -dict { PACKAGE_PIN K16   IOSTANDARD LVCMOS33 } [get_ports { sseg[2] }]; #IO_25_15 Sch=cc

set_property -dict { PACKAGE_PIN K13   IOSTANDARD LVCMOS33 } [get_ports { sseg[3] }]; #IO_L17P_T2_A26_15 Sch=cd

set_property -dict { PACKAGE_PIN P15   IOSTANDARD LVCMOS33 } [get_ports { sseg[4] }]; #IO_L13P_T2_MRCC_14 Sch=ce

set_property -dict { PACKAGE_PIN T11   IOSTANDARD LVCMOS33 } [get_ports { sseg[5] }]; #IO_L19P_T3_A10_D26_14 Sch=cf

set_property -dict { PACKAGE_PIN L18   IOSTANDARD LVCMOS33 } [get_ports { sseg[6] }]; #IO_L4P_T0_D04_14 Sch=cg

set_property -dict { PACKAGE_PIN H15   IOSTANDARD LVCMOS33 } [get_ports { sseg[7] }]; #IO_L19N_T3_A21_VREF_15 Sch=dp

set_property -dict { PACKAGE_PIN J17   IOSTANDARD LVCMOS33 } [get_ports { an[0] }]; #IO_L23P_T3_FOE_B_15 Sch=an[0]

set_property -dict { PACKAGE_PIN J18   IOSTANDARD LVCMOS33 } [get_ports { an[1] }]; #IO_L23N_T3_FWE_B_15 Sch=an[1]

set_property -dict { PACKAGE_PIN T9    IOSTANDARD LVCMOS33 } [get_ports { an[2] }]; #IO_L24P_T3_A01_D17_14 Sch=an[2]

set_property -dict { PACKAGE_PIN J14   IOSTANDARD LVCMOS33 } [get_ports { an[3] }]; #IO_L19P_T3_A22_15 Sch=an[3]

##Buttons

set_property -dict { PACKAGE_PIN N17   IOSTANDARD LVCMOS33 } [get_ports { d3 }]; #IO_L9P_T1_DQS_14 Sch=btnc

set_property -dict { PACKAGE_PIN M18   IOSTANDARD LVCMOS33 } [get_ports { d1 }]; #IO_L4N_T0_D05_14 Sch=btnu

set_property -dict { PACKAGE_PIN P17   IOSTANDARD LVCMOS33 } [get_ports { d2 }]; #IO_L12P_T1_MRCC_14 Sch=btnl

set_property -dict { PACKAGE_PIN M17   IOSTANDARD LVCMOS33 } [get_ports { d4 }]; #IO_L10N_T1_D15_14 Sch=btnr

set_property -dict { PACKAGE_PIN P18   IOSTANDARD LVCMOS33 } [get_ports { d5 }]; #IO_L9N_T1_DQS_D13_14 Sch=btnd

## Test Bench

The test-bench module tests the finite state machine (FSM) module. It can be used to assign inputs to the FSM and examine its outputs accordingly. Enables us to create different scenarios and stimulate the results.

### Scenarios

There are three possible scenarios:

- The user enters the right combinational key

Figure 4 Right from the first try

- The user enters the wrong followed by the right combinational key

Figure 5 Right from second try

- The user enters the wrong combinational key twice in a row

Figure 6 Wrong twice

# Results

Figure 7 Output 1

Figure 8 Output 2

Figure 9 Output 3

# Conclusion

The experimental results, theoretical results, and simulation results are identical as expected. Initially the digital lock displays zeros for the first, second and third digits and it diplays 1 for the number of remaining tries. Then The digital lock displays the decimal digits of the buttons as they are pressed. The number of tries decrements once the user enters the wrong lock key. The unlock led turns on when the user enters the right lock key and stays at that state no matter the buttons pressed. The lock led turns on when the user enters the wrong lock key twice in a row and stays at that state no matter the buttons pressed.